User can see performance of multiple machines at a time

Page 1

Farm operation name : Eg Tillage /Sowing/Spraying/Weeing/ Harvesting/Threshing /Grading

Machine name:

Brand Name

Specifications

(User should be able to add details of machine and save)

Tractor manufacturer name

Tractor model

Tractor registration no.

Specifications

Page 2

Operation log

| Tractor engine | On/Off |
| --- | --- |
| Tractor position | Changing |
| PTO rotation | On/Off |

| Operation turning on time | Operation turning off time | GPS location ON and boundary making start | GPS boundary tracing off |
| --- | --- | --- | --- |
| To be turned on by operator | To be turned off by operator or by system automatically if detects a)Tractor engine is off or b) tractor engine is on but tractor position is not changing and PTO also not operating |  |  |

The List of parameters will be displayed on screen based on implement selected

For example if tillage equipment is selected

| Parameter | Unit | Required /Theoretical value (to be set by operator at beginning of operation) | Real time value (average of 1 min data) | Average of real time values (average of on/off duration) | Variation in actual values (range) | Alert (blinking with colour indications |
| --- | --- | --- | --- | --- | --- | --- |
| Depth of operation | cm |  |  |  |  |  |
| Forward speed of operation | km/h |  |  |  |  |  |
| Field capacity | ha/h |  |  |  |  |  |
| Temperature |  |  |  |  |  |  |
| Vibration |  |  |  |  |  |  |

| Area coverage between on/off operation |  |
| --- | --- |
| Field capacity | ha/h |

| Area coverage between selected time duration |  |
| --- | --- |
| Field capacity | ha/h |

Soil Moisture content to be measured every time it takes break (to be tagged with time and GPS location)

Cone index /cone penetration resistance to be measured manually data to be added in system

(to be tagged with time and GPS location)

| Charges in case of custom hiring for operation between selected time duration |  |  |
| --- | --- | --- |
| Rate | Rs/h or Rs/h (depending on machine | Amount to be added by owner or operator of machine |
| Area covered /total time of operation | ha or h | System generated value (none should be able to change/edit) |
| Total payment to be made | Rs |  |

Reports

For machine owner or operator

Full day (only specific date): Total hours of use, total area covered, fuel refilling . major break down, any alert, payments received

Time specific on particular date: : Total hours of use, total area covered, fuel refilling . major break down, any alert, payments received

Date to date : Total hours of use, total area covered, fuel refilling . major break down, any alert , payments received

Time and date to time and date : Total hours of use, total area covered, fuel refilling . major break down, any alert , payments received

Fuel consumption data

Refilling data with date and time to be added by operator